## Safety data for 2-propanol

## General

Synonyms: isopropanol, isopropyl alcohol, iso-propanol, iso-propyl alcohol, IPA, sec-propanol, dimethylcarbinol, propan-2-ol. 
Molecular formula: CH3 CHOHCH3 
CAS No: 67-63-0 
EC No: 200-661-7

## Physical data

Appearance: colourless liquid with slight alcohol odour 
Melting point: -89 C 
Boiling point: 82 C 
Vapour density: 2.1 
Vapour pressure: 33 mm at 20 C 
Specific gravity: 0.79 
Flash point: 12 C 
Explosion limits: 2.0 % - 12 % 
Autoignition temperature: 425 C

## Stability

Stable. Incompatible with strong acids, strong oxidizing agents, halogens, aluminium, active halogen compounds. Regulated in UK under Highly Flammable Liquids and Liquefied Petroleum Gases Regulations 1972. Highly flammable. Vapour-air mixtures may be explosive.

## Toxicology

May be harmful by inhalation, ingestion or skin absorption. May act as an irritant. UK OES Long-term 980 mg/m3.

Toxicity data 
(The meaning of any abbreviations which appear in this section is given here.) 
ORL-RAT LD50 5045 mg kg-1 
Pimephales promelas LC50 11130 mg/l/96h 
Daphnia magna LC50 9500 mb/1/24h

Risk phrases 
(The meaning of any risk phrases which appear in this section is given here.) 
R11 R36 R37.

## Personal protection

Safety glasses. Effective ventilation.

Safety phrases 
(The meaning of any safety phrases which appear in this section is given here.) 
S7 S16 S24 S25 S26.