## Safety data for propanone

## General

Synonyms: dimethyl ketone, methyl ketone, 2-propanone, acetone, dimethylketal, pyroacetic acid 
Molecular formula: (CH3)2CO 
CAS No: 67-64-1 
EC No: 200-662-2

## Physical data

Appearance: colourless liquid with a fragrant, sweet odour 
Melting point: -95 C 
Boiling point: 56 C 
Vapour density: 2.0 
Vapour pressure: 181 mm Hg at 20 C 
Specific gravity: 0.79 
Flash point: -18 C 
Explosion limits: 2.6% - 13.0% 
Autoignition temperature: 538 C

## Stability

Stable. Incompatible with halogen acids and halogen compounds, strong bases, strong oxidizing agents, caustics, amines and ammonia, chlorine and chlorine compounds, strong acids, nitrosyl compounds. Highly flammable. Readily forms explosive mixtures with air.

## Toxicology

Harmful by inhalation, ingestion or skin absorption. Irritant. Liquid may cause permanent eye damage (corneal clouding). Contact with skin may cause defatting, leading to irritation. Long-term exposure may cause liver damage. Typical TLV 750 ppm.

Toxicity data 
(The meaning of any abbreviations which appear in this section is given here.) 
UNR-MAN LDLO 1159 mg kg-1 
ORL-RAT LD50 5800 mg kg-1 
UHL-RAT LC50 50 mg l-1 
IHL-RAT LC50 50100 mg/m3/8h 
ORL-MUS LD50 3000 mg kg-1 
IHL-MUS LC50 44 g/m3/4h

Irritation data 
(The meaning of any abbreviations which appear in this section is given here.) 
EYE-HMN 500 ppm 
SKN-RBT 395 mg open mld 
EYE-RBT 20 mg sev

Ecological information 
Biological degradability: good. Aquatic toxicity: low. Bioaccumulation potential: low. Fish toxicity LC50 (L. macrochirus) 8300 mg/l/96h.

Risk phrases 
(The meaning of any risk phrases which appear in this section is given here.) 
R11 R36 R37 R38 R66 R67.

## Transport information

(The meaning of any UN hazard codes which appear in this section is given here.) 
UN No 1090. Major hazard class 3.0. Packing group III. Transport category 2. EMS No 2.0-06.

## Personal protection

Safety glasses. Effective ventilation. Remove sources of ignition from the working area. Nitrile gloves.

Safety phrases 
(The meaning of any safety phrases which appear in this section is given here.) 
S9 S16 S23 S26 S33.